TECHNICAL FORM OPEN 50 WIND





DESIGNER:  Felci Yachts


LAUNCH:  mai 2000


BUILDERS:  Dolphin (Roma) and SC (Latina) 


MATERIALS:  Kevlar-glass epoxy for the hull; carbon epoxy for the deck, bulkheads and transversal and longitudinal 


reinforcements; sandwich core PVC Klegecell


COMPARTMENTS:  8 (7 watertight bulkhead)


RESULTS:  4th class 2 Transat Europe 1-New Man Star 2000


                      15th Vendée Globe 2000-2001


                       11th overall Barcolana 2001 (Trieste) and 2nd classe 0


MILES:  44.000


FLAG:  Italian


LOA:  15,24 m (+1,45 m bowsprit)


LWL:  15,24 m


BEAM:  4,73 m


DRAFT:  3.80 m


DISPLACEMENT:  7 t


BULB:  2,2 t


KEEL:  0,7 t (Weldox 900, originally canting 35°, now fixed)


HYDRAULIC:  Caritec, complete system for the canting keel


ENGINE:  Lombardini Marine 27 CV, S-drive Gori propeller 


GENERATOR:  1 standard 65 Ah + 1 Master Volt 135 Ah


BATTERY:  2x175 Ah + 1x75 Ah (12v)


WIND GENERATOR:  1 Air Marine


SOLAR PANELS:  2x32 W (for emergency only)


RUDDERS:   2 external on the stern + 1 emergency


DAGGERBOARD:  1


SAIL AREA (upwind):  220 m2


SAIL AREA (downwind):  380 m2


SAILS (North Italy):     MAIN SAIL - spectra 120 m2


STORM  MAIN SAIL - dacron 20 m2 


LIGHT GENOA - kevlar (head) 100 m2 ( furler Facnor)


HEAVY GENOA - spectra (7/8) 65 m2 (with Profurl)


JIB - spectra (on kevlar inner forestay) 45 m2


STAYSAIL - spectra (on kevlar baby) 22 m2


STORM JIB - dacron 11 m2 


GENAKER - (7/8)  180 m2


HEAD GENAKER - 260 m2


MAST:  22,2 m, carbon high module, Sparcraft USA


RIG:  7/8, 3 spreaders, rod, kevlar runners


AUTOPILOTS:  2 Autohelm 6000, 1 Autohelm 4000


WATERMAKER:  Power Survyvor 40


EPIRB:  2 Sarsat ACR


LIFERAFTS:  1 Zodiac Solas 4 places, 1 BF 6 pl., 1 BF 4pl.


COMMUNICATION:  VHF Skanti, BLU Icom, Standard C Trimble Galaxy, Standard M Thrane&Thrane


POSITION:  Standard C, GPS (fixed) Raytheon, Garmin mobile


RADAR AND  ELECTRONICS:  Raytheon


METEO:  Meteofax Furuno


PC:   Toshiba


NAVIGATION SOFTWARE: SW&N





